OPERATIONS & SUPPLY CHAIN

CHEAT HANDBOOK

Formulas, Concepts & Quick Reference

BCOR 440 / IE425 • Spring 2026

| ZONE 1 How We Make Things | ZONE 2 Where Stuff Comes From | ZONE 3 How We Plan | ZONE 4 How We Optimize |
| --- | --- | --- | --- |
| Ch 7-8: Processes & Layout Ch 4: Projects Ch 5: Capacity Module 1 | Ch 14: Lean Ch 15: Logistics Ch 16: Sourcing Module 2 | Ch 18: Forecasting Ch 19: S&OP Ch 20-21: Inventory & MRP Modules 3-4 | Ch 12-13: Quality Ch 22: Scheduling Ch 22S: Constraints Modules 5-6 |

ZONE 1: HOW WE MAKE THINGS

Processes, Projects, Manufacturing, Capacity | Chapters 4-8 | Module 1

CAPACITY MANAGEMENT

Chapter 5

| Capacity Utilization | Utilization = Output ÷ Design Capacity × 100% How much of your maximum ability you're actually using Use when: Measuring how efficiently you're using resources |
| --- | --- |

| Efficiency | Efficiency = Actual Output ÷ Effective Capacity × 100% How well you're hitting realistic targets (not theoretical max) Use when: Comparing actual performance to expected performance |
| --- | --- |

💡 Design Capacity: Theoretical maximum output (if everything goes perfectly)

💡 Effective Capacity: Realistic output accounting for maintenance, breaks, changeovers

💡 Capacity Cushion: Extra capacity kept as buffer (typically 10-20% for manufacturing, higher for services)

| Capacity Strategy | When to Use |
| --- | --- |
| Lead Strategy | Build capacity BEFORE demand (risky but captures market share) |
| Lag Strategy | Build capacity AFTER demand proven (safer but may lose customers) |
| Match Strategy | Add capacity in small increments to match demand |

MANUFACTURING PROCESSES

Chapter 7 | Practice Operations: Module 1

| Process Type | Characteristics |
| --- | --- |
| Job Shop | Low volume, high variety, flexible, skilled workers (custom furniture) |
| Batch | Medium volume, moderate variety (bakery, clothing) |
| Assembly Line | High volume, low variety, sequential steps (cars, electronics) |
| Continuous Flow | Very high volume, no variety, runs 24/7 (oil refinery, paper mill) |

💡 Product-Process Matrix: Match process type to volume/variety—moving diagonal is most efficient

💡 Make-to-Order (MTO): Start production only after customer orders (custom products)

💡 Make-to-Stock (MTS): Produce to inventory based on forecasts (standard products)

💡 Assemble-to-Order (ATO): Keep components ready, customize assembly when ordered (computers)

PROCESS ANALYSIS

Chapter 11

| Cycle Time | Cycle Time = Time Available ÷ Units Required Maximum time allowed at each station to meet demand Use when: Designing assembly lines or calculating takt time |
| --- | --- |

| Throughput Rate | Throughput = 1 ÷ Bottleneck Time Units per time period the system can produce (limited by slowest step) Use when: Finding maximum system output |
| --- | --- |

| Little's Law | Inventory = Throughput × Flow Time Average items in system = rate × time each item spends in system Use when: Relating inventory, throughput, and lead time |
| --- | --- |

💡 Bottleneck: The step with the longest processing time—determines system capacity

💡 Flow Time: Total time from start to finish for one unit

💡 Takt Time: The "beat" of production—pace needed to meet customer demand

PROJECT MANAGEMENT

Chapter 4

| Expected Time (PERT) | TE = (a + 4m + b) ÷ 6 a=optimistic, m=most likely, b=pessimistic; weighted average Use when: Estimating task duration with uncertainty |
| --- | --- |

| Variance (PERT) | σ² = [(b - a) ÷ 6]² Spread of time estimates squared Use when: Calculating risk/uncertainty in project timing |
| --- | --- |

| Project Z-Score | Z = (Due Date - Expected Completion) ÷ √(Sum of Critical Path Variances) How many standard deviations between target and expected date Use when: Finding probability of finishing on time |
| --- | --- |

| EVM Metric | Formula & Meaning |
| --- | --- |
| Cost Variance (CV) | CV = EV - AC (positive = under budget) |
| Schedule Variance (SV) | SV = EV - PV (positive = ahead of schedule) |
| Cost Performance Index | CPI = EV ÷ AC (>1 = efficient) |
| Schedule Performance Index | SPI = EV ÷ PV (>1 = ahead) |

💡 Critical Path: Longest path through project network—determines minimum project duration

💡 Slack/Float: Time an activity can be delayed without delaying the project (Critical Path has 0 slack)

💡 Crashing: Adding resources to reduce time on critical path activities (usually increases cost)

ZONE 2: WHERE STUFF COMES FROM

Supply Chain, Sourcing, Lean, Logistics | Chapters 14-16 | Module 2

LEAN SUPPLY CHAINS

Chapter 14 | Practice Operations: Module 2

| The 7 Wastes (TIMWOOD) | Examples |
| --- | --- |
| Transportation | Moving materials unnecessarily between locations |
| Inventory | Excess stock sitting idle, tying up cash |
| Motion | Unnecessary worker movement (poor layout) |
| Waiting | Idle time—machines, workers, or materials |
| Overproduction | Making more than needed, before needed |
| Overprocessing | More work than customer values (gold-plating) |
| Defects | Errors requiring rework or scrap |

💡 JIT (Just-in-Time): Receive materials only when needed for production—minimizes inventory

💡 Kanban: Visual signal (card/bin) that triggers replenishment—pull system

💡 Pull System: Production driven by actual demand, not forecasts

💡 Push System: Production driven by forecasts—make and hope to sell

💡 Single Minute Exchange of Dies (SMED): Reduce changeover time to enable smaller batches

💡 5S: Sort, Set in order, Shine, Standardize, Sustain—workplace organization

💡 Poka-Yoke: Mistake-proofing—design prevents errors

💡 Kaizen: Continuous improvement through small, incremental changes

LOGISTICS & DISTRIBUTION

Chapter 15

| Transport Mode | Best For |
| --- | --- |
| Truck | Flexible, door-to-door, medium distances, most freight |
| Rail | Heavy/bulk goods, long distances, cost-effective |
| Air | High-value, time-sensitive, perishable goods |
| Water/Ship | Very heavy/bulky, international, lowest cost per ton-mile |
| Pipeline | Continuous flow commodities (oil, gas, water) |

💡 Cross-Docking: Unload from inbound trucks → sort → load directly to outbound (no storage)

💡 Milk Run: Single vehicle picks up from multiple suppliers on one route

💡 3PL: Third-party logistics—outsourcing warehousing and transportation

💡 Postponement: Delay final customization until customer order (reduces inventory risk)

GLOBAL SOURCING & PROCUREMENT

Chapter 16 | Practice Operations: Module 2

| Total Cost of Ownership | TCO = Purchase Price + Ordering Costs + Holding Costs + Quality Costs + Risk Costs True cost goes beyond sticker price Use when: Comparing suppliers or make vs. buy decisions |
| --- | --- |

| Supplier Relationship Type | Characteristics |
| --- | --- |
| Transactional | Price-focused, short-term, easy to switch |
| Collaborative | Long-term partnership, shared info, mutual benefit |
| Strategic Alliance | Deep integration, joint investment, shared risk/reward |

💡 Single Sourcing: One supplier for a part—builds relationship but creates risk

💡 Multiple Sourcing: Several suppliers—reduces risk but harder to manage

💡 Bullwhip Effect: Small demand changes at retail amplify up the supply chain (causes inventory swings)

💡 Landed Cost: Total cost including shipping, duties, taxes, handling to get item to destination

ZONE 3: HOW WE PLAN

Forecasting, Inventory, MRP, S&OP | Chapters 18-21 | Modules 3-4

FORECASTING

Chapter 18 | Practice Operations: Module 3

| Simple Moving Average | Forecast = (Sum of last n periods) ÷ n Average of recent periods; smooths random variation Use when: Stable demand with no trend |
| --- | --- |

| Weighted Moving Average | Forecast = Σ(Weight × Demand for each period) Recent periods get higher weights; weights must sum to 1.0 Use when: Stable demand, want more recent data influence |
| --- | --- |

| Exponential Smoothing | Ft+1 = α × Dt + (1 - α) × Ft New forecast = α(actual) + (1-α)(old forecast); α = smoothing constant (0.1-0.3 typical) Use when: Quick calculation, self-correcting |
| --- | --- |

| Linear Regression | Y = a + bX a = intercept, b = slope; use for trends Use when: Clear upward or downward trend |
| --- | --- |

| MAD (Mean Absolute Deviation) | MAD = Σ¦Actual - Forecast¦ ÷ n Average forecast error (ignoring + or -) Use when: Measuring forecast accuracy |
| --- | --- |

| MAPE (%) | MAPE = [Σ(¦Actual - Forecast¦ ÷ Actual)] ÷ n × 100% Average error as percentage of actual Use when: Comparing accuracy across different-sized items |
| --- | --- |

💡 Tracking Signal: TS = Running Sum of Errors ÷ MAD (should stay between ±4; if not, forecast is biased)

💡 Seasonal Index: Actual ÷ Trend value; >1 means above-average season, <1 means below

SALES & OPERATIONS PLANNING

Chapter 19 | Practice Operations: Module 4

| Strategy | How It Works |
| --- | --- |
| Level Strategy | Constant workforce/output; absorb demand swings with inventory |
| Chase Strategy | Match production to demand; hire/fire or use overtime |
| Mixed Strategy | Combine approaches based on cost trade-offs |

💡 Aggregate Planning: Medium-term (6-18 months) decisions about workforce size, production rates, inventory

💡 Yield Management: Adjusting prices to match demand to fixed capacity (airlines, hotels)

💡 Key Trade-offs: Hiring/firing costs vs. inventory costs vs. overtime vs. stockout costs

INVENTORY MANAGEMENT

Chapter 20

| EOQ (Economic Order Quantity) | Q* = √(2DS ÷ H) D = annual demand, S = order/setup cost, H = holding cost per unit per year Use when: Finding order quantity that minimizes total inventory costs |
| --- | --- |

| Reorder Point | ROP = d × L + Safety Stock d = demand per day, L = lead time in days Use when: When to place an order |
| --- | --- |

| Safety Stock | SS = z × σL or SS = z × σd × √L z = service level z-score, σ = standard deviation of demand or lead time Use when: Buffer against demand or lead time uncertainty |
| --- | --- |

| Total Annual Cost | TC = DC + (D/Q)S + (Q/2)H Purchase cost + Ordering cost + Holding cost Use when: Evaluating inventory policies |
| --- | --- |

| Newsvendor (Single Period) | Order until P(Demand ≤ Q) = Cu ÷ (Cu + Co) Cu = cost of understocking, Co = cost of overstocking Use when: One-time orders with uncertain demand (perishables, fashion) |
| --- | --- |

| ABC Classification | Policy |
| --- | --- |
| A Items (70-80% of $) | Tight control, frequent review, accurate records |
| B Items (15-20% of $) | Moderate control, periodic review |
| C Items (5-10% of $) | Simple controls, large order quantities |

💡 Fixed-Order Quantity System (Q): Order same quantity when inventory hits reorder point

💡 Fixed-Time Period System (P): Review inventory at fixed intervals, order variable quantity to reach target

MATERIAL REQUIREMENTS PLANNING (MRP)

Chapter 21 | Practice Operations: Module 4

| Net Requirements | Net Req = Gross Req - Scheduled Receipts - On Hand + Safety Stock What you actually need to order/produce Use when: MRP explosion calculations |
| --- | --- |

💡 Bill of Materials (BOM): Recipe showing all components and quantities needed to make parent item

💡 Master Production Schedule (MPS): What end items to make, in what quantities, when—drives MRP

💡 Lot-for-Lot: Order exactly what's needed each period (minimizes inventory, maximizes orders)

💡 Lead Time Offsetting: Schedule release date = due date minus lead time

💡 Low-Level Coding: Assign each item to its lowest level in any BOM (ensures proper explosion)

💡 Planned Order Release: When to release order to meet planned receipt date

ZONE 4: HOW WE OPTIMIZE

Quality, Scheduling, Constraints | Chapters 12-13, 22 | Modules 5-6

QUALITY MANAGEMENT & STATISTICAL PROCESS CONTROL

Chapter 12-13

| Process Capability (Cp) | Cp = (USL - LSL) ÷ 6σ Can the process fit within spec limits? Cp ≥ 1.33 is good Use when: Assessing whether process CAN meet specs |
| --- | --- |

| Process Capability Index (Cpk) | Cpk = min[(USL - μ) ÷ 3σ, (μ - LSL) ÷ 3σ] Is the process centered AND capable? Cpk ≥ 1.33 is good Use when: Assessing whether process IS meeting specs |
| --- | --- |

| X-bar Chart Limits | UCL = X̄ + A₂R̄ LCL = X̄ - A₂R̄ Control limits for sample means; A₂ from table based on sample size Use when: Monitoring process center |
| --- | --- |

| R Chart Limits | UCL = D₄R̄ LCL = D₃R̄ Control limits for sample ranges; D₃, D₄ from table Use when: Monitoring process variation |
| --- | --- |

| p-Chart Limits | UCL = p̄ + 3√(p̄(1-p̄)/n) LCL = p̄ - 3√(p̄(1-p̄)/n) For proportion defective; n = sample size Use when: Monitoring defect rates (attributes data) |
| --- | --- |

| c-Chart Limits | UCL = c̄ + 3√c̄ LCL = c̄ - 3√c̄ For count of defects per unit; c̄ = average defects Use when: Monitoring defect counts (scratches, blemishes) |
| --- | --- |

💡 In Control: All points between control limits, no patterns—only common cause variation

💡 Out of Control: Points outside limits OR patterns (7 consecutive up/down, 7 above/below center)

💡 Six Sigma: Process with 3.4 defects per million opportunities (6σ from mean to spec limit)

Common Control Chart Constants (for reference):

| n | A₂ | D₃ | D₄ | d₂ |
| --- | --- | --- | --- | --- |
| 2 | 1.880 | 0 | 3.267 | 1.128 |
| 3 | 1.023 | 0 | 2.574 | 1.693 |
| 4 | 0.729 | 0 | 2.282 | 2.059 |
| 5 | 0.577 | 0 | 2.114 | 2.326 |

WORKCENTER SCHEDULING

Chapter 22 | Practice Operations: Modules 5-6

| Priority Rule | How It Works |
| --- | --- |
| FCFS (First Come First Serve) | Process in arrival order (fair but not optimal) |
| SPT (Shortest Processing Time) | Shortest job first (minimizes avg flow time & WIP) |
| EDD (Earliest Due Date) | Earliest deadline first (minimizes max lateness) |
| CR (Critical Ratio) | CR = Time Remaining ÷ Work Remaining (<1 = behind) |
| LPT (Longest Processing Time) | Longest job first (for parallel machines) |

| Flow Time | Flow Time = Wait Time + Processing Time Total time job spends in the system Use when: Measuring shop performance |
| --- | --- |

| Lateness | Lateness = Completion Time - Due Date Positive = late, Negative = early Use when: Tracking on-time delivery |
| --- | --- |

| Critical Ratio | CR = (Due Date - Today) ÷ Remaining Processing Time CR < 1: behind schedule; CR = 1: on track; CR > 1: ahead Use when: Dynamic scheduling decisions |
| --- | --- |

💡 Makespan: Total time to complete all jobs (finish time of last job minus start)

💡 WIP (Work in Process): Jobs currently in the system (waiting or being processed)

💡 Johnson's Rule: 2-machine sequencing: find smallest time → if Machine 1, schedule first; if Machine 2, schedule last; repeat

THEORY OF CONSTRAINTS

Chapter 22S | Practice Operations: Modules 5-6

The Five Focusing Steps:

- IDENTIFY the system's constraint (bottleneck)

- EXPLOIT the constraint (maximize its output—no idle time!)

- SUBORDINATE everything else (pace all work to the bottleneck)

- ELEVATE the constraint (add capacity to bottleneck if needed)

- REPEAT—find the new constraint

💡 Drum-Buffer-Rope: Drum = bottleneck pace; Buffer = time protection; Rope = release materials at bottleneck rate

💡 Throughput: Rate at which system generates money through sales (revenue minus materials)

💡 Constraint: Resource with capacity ≤ demand placed on it—limits system output

QUICK REFERENCE: Z-SCORES & SERVICE LEVELS

| Service Level | Z-Score | Meaning |
| --- | --- | --- |
| 50% | 0.00 | Stockout 50% of cycles |
| 75% | 0.67 | Stockout 25% of cycles |
| 84% | 1.00 | 1 standard deviation buffer |
| 90% | 1.28 | Common target level |
| 95% | 1.65 | High service (retail) |
| 97.5% | 1.96 | ≈2 sigma one-tail |
| 99% | 2.33 | Premium service (A items) |

PRACTICE OPERATIONS SIMULATION MAPPING

| Module | Focus | Key Skills & Chapters |
| --- | --- | --- |
| 1 | Production Process | Job shop layout, production flow, scheduling (Ch 7-8) |
| 2 | Managing Suppliers | Purchasing, inventory control, capital equipment (Ch 14-16, 20) |
| 3 | Forecasting & Contracts | Bidding strategy, demand forecasting, make-to-order (Ch 2-3, 18) |
| 4 | HR & Capacity Planning | Hiring, training, capacity decisions, learning curves (Ch 5-6, 19) |
| 5-6 | Full Integration | Apply ALL skills: bottlenecks, scheduling, maximize net worth (Ch 22, 22S) |

Remember: Understanding beats memorizing. Use this sheet to

look up formulas so you can focus on WHEN and WHY to use them.