Analysis of Bakery Process

ANSWER KEY with Explanations

(Adapted from Wharton, UPenn)

Context: This bakery makes croissants in batches of 50. It's a make-to-order shop — no orders, no production. Let's figure out the bottleneck, capacity, and utilization.

The Key Insight for Process Analysis In any process, your capacity is limited by your SLOWEST step (the bottleneck). It doesn't matter how fast everything else is — you can only go as fast as your slowest station.

## Step 1: Map Out the Process Times

| Station | Time/Batch | Batches/Hour | Path |
| --- | --- | --- | --- |
| Mix Dough | 5 min | 12 | Dough |
| Proof | 15 min | 4 | Dough |
| Roll & Cut | 5 min | 12 | Dough |
| Mix Filling | 10 min | 6 | Filling |
| Wait for Filling | 15 min | 4 | Filling |
| Fill & Fold | 5 min | 12 | Combined |
| Bake | 20 min | 3 | Combined |
| Pack | 10 min | 6 | Combined |

# Question 1: What's the Bottleneck?

Answer: BAKE (20 min/batch = only 3 batches/hour)

How to Find the Bottleneck Look for the station with the LONGEST processing time (or equivalently, the LOWEST capacity per hour). Here, Bake takes 20 minutes per batch — that's slower than any other station in the combined path.

What About Proof and the Filling Wait? Both take 15 min, but they're on PARALLEL paths. The dough path and filling path run simultaneously, so only one of them matters for overall timing. Once they merge at Fill & Fold, Bake becomes the constraint.

# Question 2: Daily Capacity

If the oven runs 8 hours a day, what's our maximum output?

Capacity = Bottleneck Rate × Hours = 3 batches/hr × 8 hrs = 24 batches/day That's 24 batches × 50 croissants = 1,200 croissants/day maximum

Answer: 24 batches/day (or 1,200 croissants)

# Question 3: Demand of 1,000 Croissants (20 batches)

## 3.1 What is the Flow Rate?

Flow Rate Definition Flow Rate = the actual throughput of the system. It's the MINIMUM of capacity and demand. You can't produce more than you can make (capacity), and you won't produce more than customers want (demand).

Flow Rate = min(Capacity, Demand) = min(24, 20) = 20 batches/day We CAN make 24, but we only NEED 20

Answer: 20 batches/day

## 3.2 Supply-Constrained or Demand-Constrained?

Answer: Demand-Constrained

| Demand-Constrained | Supply-Constrained |
| --- | --- |
| Demand < Capacity We could make more, but nobody wants it 20 < 24 ← THIS ONE | Demand > Capacity Customers want more than we can make We're leaving money on the table |

## 3.3 Process Utilization

Utilization = How Hard Are We Working? Utilization measures how much of our capacity we're actually using. It's always expressed as a percentage. High utilization (>85%) means you're efficient but might struggle with demand spikes. Low utilization means you have room to grow.

Utilization = Flow Rate / Capacity = 20 / 24 = 0.833 = 83.3% We're using about 83% of our oven capacity

Answer: 83.3%

# Question 4: Implied Utilization (Demand = 28 batches)

Now demand jumps to 1,400 croissants (28 batches). What's the implied utilization?

Implied vs. Actual Utilization — Know the Difference! Implied utilization asks: What would utilization BE if we could meet all demand? It can exceed 100%, which means we CAN'T actually meet demand. Actual utilization can never exceed 100%.

Implied Utilization = Demand / Capacity = 28 / 24 = 1.167 = 116.7% Demand exceeds capacity — we can't keep up!

Answer: 116.7% (we're 4 batches short!)

What Does 116% Implied Utilization Mean in Practice? It means we'd need 116% of our current oven capacity to meet demand. Options: (1) Add a second oven, (2) Run longer hours, (3) Turn away customers, (4) Speed up baking (if possible). This is exactly the kind of trade-off managers face every day.

# Quick Reference: The Formulas

| Concept | Formula | Units |
| --- | --- | --- |
| Capacity | Bottleneck rate × Time | units/time |
| Flow Rate | min(Capacity, Demand) | units/time |
| Utilization | Flow Rate / Capacity | % (max 100%) |
| Implied Utilization | Demand / Capacity | % (can exceed 100%) |