BCOR-440 / IE-425

Operations & Supply Chain Management

Quiz 3 — Study Guide

Spring 2026  •  Dr. Yaa

Chapters 12, 13 (with light review of Chs 17, 22, and 22s)

Know these concepts cold — they come up in interviews and on the job. This study guide covers everything Quiz 3 will test, organized for quick recall. Each “Interview Tip” shows you how to phrase the concept under pressure. Don’t just memorize — practice saying these out loud.

Chapter 12: Quality Management

What Is Quality?

Design quality:  The inherent value of the product in the marketplace (features, performance, aesthetics).

Conformance quality:  The degree to which the product meets its design specifications.

Remember: a product can have high design quality (luxury car) but low conformance quality (frequent breakdowns), or vice versa.

The Four Costs of Quality

| Cost Type | What It Is | Example |
| --- | --- | --- |
| Prevention | Costs to prevent defects from occurring | Employee training, quality planning |
| Appraisal | Costs of inspection and testing | Incoming materials inspection, audits |
| Internal Failure | Costs for defects caught BEFORE reaching the customer | Scrap, rework, retesting |
| External Failure | Costs for defects that reach the customer | Recalls, warranty claims, lawsuits |

Interview Tip If an interviewer asks about cost of quality, emphasize that investing in prevention saves roughly 10x in failure and appraisal costs. That shows you understand the strategic side, not just the categories.

The Quality Gurus — Know Who Said What

|  | Crosby | Deming | Juran |
| --- | --- | --- | --- |
| Definition | Conformance to requirements | Predictable uniformity & dependability at low cost | Fitness for use |
| Approach | Prevention, not inspection; zero defects | Reduce variability via continuous improvement; 14 points | General management approach; human elements |
| Worker responsibility | Responsible for quality (rejected stat. acceptable levels) | 9.4% of problems due to workers | < 20% of problems due to workers |

DMAIC Cycle (Six Sigma’s Problem-Solving Method)

| Phase | Purpose | Common Tools |
| --- | --- | --- |
| Define | Identify customers and their priorities | Project charter, VOC, CTQ |
| Measure | Determine how to measure the process and how it’s performing | Data collection, check sheets, process mapping |
| Analyze | Determine the most likely causes of defects | Fishbone diagram, Pareto chart, 5 Whys |
| Improve | Identify means to remove the causes of defects | DOE, poka-yoke, pilot testing |
| Control | Determine how to maintain the improvements | Control charts, SPC, standard work |

Other Key Ch 12 Terms

Six Sigma:  A philosophy and set of methods to eliminate defects. Goal: 3.4 DPMO.

TQM (Total Quality Management):  Managing the entire organization to excel on all quality dimensions important to the customer.

Lean Manufacturing:  Achieves high-volume production with minimal waste using JIT methods. Uses PDCA cycle.

Lean Six Sigma:  Combines Six Sigma’s quality tools with Lean’s waste-elimination focus.

Kaizen:  Continuous improvement through small, incremental changes.

Poka-Yoke:  Mistake-proofing — any device or method that prevents errors before they occur (e.g., a USB plug that only fits one way; a fixture that physically blocks an incorrect assembly).

FMEA:  A structured approach to identify, estimate, and prioritize risk of possible failures at each process stage.

DOE (Design of Experiments):  A statistical method to determine cause-and-effect relationships between process variables.

Seven Basic Quality Tools:  Flowchart, check sheet, histogram, Pareto chart, scatter diagram, control chart, fishbone diagram.

Five Whys:  A root-cause technique that asks “Why?” repeatedly to dig past symptoms to underlying causes.

ISO 9000 / ISO 14000:  International quality and environmental management standards. Three certification forms: first party (self-audit), second party (customer audits supplier), third party (external certified agency).

Malcolm Baldrige Award:  The U.S. national quality award, administered by NIST.

Quality at the Source:  The person doing the work is responsible for ensuring specs are met.

The 7 Wastes (Muda / TIMWOOD):  Transport, Inventory, Motion, Waiting, Overproduction, Over-processing, Defects.

Chapter 13: Statistical Quality Control (SPC)

Two Types of Variation

Assignable (Special) Variation:  Caused by identifiable factors that can be managed (e.g., machine malfunction, untrained operator). Should be investigated.

Common (Random) Variation:  Inherent in the process itself. Cannot be eliminated without fundamentally changing the process.

Interview Tip In an interview, you might say: “Control charts help us separate signal from noise — assignable variation is the signal we need to act on.”

Control Charts — Which One to Use?

| Chart | Data Type | Scale | When to Use | Example |
| --- | --- | --- | --- | --- |
| X-bar & R | Variable | Continuous | Measuring a quantity (weight, length, temp) | Coca-Cola bottle weight |
| p-chart | Attribute | Binary (yes/no) | Proportion defective in a sample | Patient satisfaction (yes/no) |
| c-chart | Attribute | Count | Number of defects when total opportunities unknown | Delayed flights per day |

Rule of thumb: p-charts need larger sample sizes (aim for at least 1 defective per sample). X-bar charts work with smaller samples.

Interpreting Control Charts

A process is “in control” when all points fall within the UCL and LCL with no nonrandom patterns. Watch for these red flags:

- Point outside limits: investigate immediately.

- Run of 8+ points above or below the center line: investigate for a sustained shift in the process, even if all points are inside the control limits.

- Trend in one direction: investigate for progressive change (tool wear, drift).

- Erratic behavior: investigate the source of instability.

Process Capability: Cp vs. Cpk

| Measure | What It Tells You | Key Distinction |
| --- | --- | --- |
| Cp | Can the process fit within the spec limits, assuming it’s perfectly centered? | Only measures spread — ignores where the mean actually is. |
| Cpk | Can the process actually produce within spec limits given where the mean is? | Accounts for both spread AND centering. |

Cpk = 1.0 means the process barely fits. Cpk > 1.0 means there’s room to spare. Cpk < 1.0 means the process WILL produce defects outside specs.

If Cp = Cpk, the process is perfectly centered. If Cp > Cpk, the process is off-center.

Interview Tip A classic interview question: “What’s the difference between Cp and Cpk?” Answer: “Cp tells you IF the process can fit within specs assuming perfect centering. Cpk tells you if it ACTUALLY does, given where the mean is.” Bonus: if you’re given Cp = 1.5 and Cpk = 0.8, the right move is to re-center the process — not to reduce variation. The spread is fine; the mean has drifted.

Other Key Ch 13 Terms

Acceptance Sampling:  Inspecting a sample from a batch of existing goods to decide whether to accept or reject the entire lot.

AQL (Acceptable Quality Level):  Maximum acceptable percentage of defectives defined by the producer.

LTPD (Lot Tolerance Percent Defective):  The defect percentage at which the consumer would reject.

Producer’s Risk (α):  Probability of rejecting a good lot (Type I error).

Consumer’s Risk (β):  Probability of accepting a bad lot (Type II error).

Attribute vs. Variable Data:  Attribute = pass/fail or count (use p-chart or c-chart). Variable = continuous measurement (use X-bar and R-chart).

Factors Table (Exhibit 13.7):  When the true process standard deviation is unknown, you compute X-bar and R-chart control limits using A2, D3, and D4 constants from a factors table that depend on sample size n. You don’t need to memorize the values — just know that’s where they come from.

Chapters 17, 22, & 22s: Light Review

These chapters are lightly tested on Quiz 3 — just a handful of questions. Know the headline terms below cold.

Chapter 17: ERP & IoT

ERP (Enterprise Resource Planning):  A system that integrates all functional areas (finance, manufacturing/logistics, sales/marketing, HR) through shared information in a single database.

Largest ERP vendor:  SAP.

Four typical ERP module categories:  Finance; Manufacturing & Logistics (usually the largest and most complex); Sales & Marketing; Human Resources.

SAP’s four supply chain functions:  Planning, Execution, Collaboration, Coordination.

Transaction processing:  Posting and tracking the activities that document a business.

Collaborative manufacturing:  Sharing information with supply chain partners to coordinate production and improve visibility.

Chapter 22: Workcenter Scheduling

Four standard measures of schedule performance:  (1) Meeting due dates, (2) minimizing flow time, (3) minimizing WIP inventory, (4) minimizing idle time. Note: maximizing machine utilization is NOT one of them — in fact, max utilization at non-bottlenecks just builds WIP.

Scheduling involves:  (1) Allocating jobs to resources, (2) sequencing jobs, (3) dispatching/releasing jobs, (4) monitoring job status.

Chapter 22s: Theory of Constraints (TOC)

Bottleneck (constraint):  The slowest workstation in the system. Determines the throughput of the entire system.

Core TOC insight:  An hour saved at the bottleneck is an hour saved for the whole system. An hour saved anywhere else is a mirage.

Five Focusing Steps:  (1) Identify the constraint, (2) exploit it, (3) subordinate everything else to it, (4) elevate it, (5) if the constraint moves, repeat.

Interview Tip TOC shows up in operations and consulting interviews constantly. If asked “how would you improve a slow process?” the right move is: find the bottleneck first, then focus all improvement effort there. Don’t optimize the parts that are already fast — it just makes the queue worse.

Quick Self-Check Questions

Cover the answers and see if you can respond confidently before reading them.

1. A pharmaceutical company finds contaminated pills during a final inspection before they ship to pharmacies. Is this an internal or external failure cost?

Answer: Internal failure — the defect was caught before reaching the customer.

2. You are measuring the diameter (in mm) of ball bearings in samples of 6. What control chart(s) do you use?

Answer: X-bar and R-chart — you’re measuring a continuous variable.

3. A hotel tracks the proportion of guests who report a problem with their room each night. What chart?

Answer: p-chart — attribute data with a binary outcome (problem / no problem).

4. Your process has Cp = 1.5 and Cpk = 0.9. What does this tell you?

Answer: The process has enough spread to fit within specs (Cp > 1), but it’s not centered — the mean has drifted toward one spec limit, making it not capable (Cpk < 1). The fix is to re-center, not to reduce variation.

5. Which quality guru believed in “zero defects” and rejected statistically acceptable levels of quality?

Answer: Philip Crosby.

6. What’s the difference between Lean and Six Sigma?

Answer: Lean focuses on eliminating waste and uses the PDCA cycle. Six Sigma focuses on reducing variation/defects and uses DMAIC. Lean Six Sigma combines both approaches.

7. Name the seven basic quality tools.

Answer: Flowchart, check sheet, histogram, Pareto chart, scatter diagram, control chart, cause-and-effect (fishbone) diagram.

8. A control chart shows 8 consecutive points above the centerline, all inside the control limits. Is the process in control?

Answer: No — a long run on one side of the center line (typically 8+ points) signals a sustained process shift that should be investigated, even if no point has crossed a control limit.

9. What does FMEA stand for, and which DMAIC phase would you most likely use it in?

Answer: Failure Mode and Effect Analysis. Most commonly used in the Analyze and Improve phases.

10. A company receives a shipment of 20,000 screws. They want to inspect a sample and decide whether to accept or reject the lot. What method is this?

Answer: Acceptance sampling.

11. What are the four typical functional areas integrated by an ERP system?

Answer: Finance; Manufacturing & Logistics; Sales & Marketing; Human Resources.

12. In Theory of Constraints, what determines the throughput of a system?

Answer: The bottleneck (constraint) — the slowest workstation. Improving non-bottleneck workstations doesn’t increase system throughput; it just builds WIP.

Good luck — you’ve got this! Remember: this isn’t just exam prep. Every term here is interview-ready vocabulary you’ll use in your career.